Korg MS-20 Architecture & DSP Engineering Specification

Digital Emulation Blueprint: TPT Sallen-Key Filters, PolyBLEP VCOs, IC Topologies & Rust Multi-Target Architecture

This document details the complete mathematical models, component-level IC topologies, and software architecture required to implement an authentic digital emulation of the classic Korg MS-20 synthesizer in Rust, targeting both a native Text User Interface (TUI via ratatui/crossterm) and a Web target (AudioWorklet via WebAssembly).

1. Core Synthesis Engine Architecture

The MS-20 is a monophonic, dual-VCO, dual-VCF semi-modular synthesizer using linear Hz/V pitch tracking and S-Trig switching in the analog domain. In DSP, signal calculation is carried out in normalized floating-point samples (f32 or f64). The architecture operates in a strictly monophonic voice allocation mode with high-priority or low-priority note tracking.

+-----------+ ¦ VCO 1 ¦----+ +-----------+ ¦ ¦ (Sync/RM)¦ +--------------+ +--------------+ +---------+ v +-->¦ KORG35 HPF ¦-->¦ KORG35 LPF ¦-->¦ VCA ¦--> Output +-----------+ ¦ ¦ 12dB/oct TPT ¦ ¦ 12dB/oct TPT ¦ ¦ (Linear/¦ ¦ VCO 2 ¦----+ +--------------+ +--------------+ ¦ Exp) ¦ +-----------+ ^ ^ +---------+ ¦ ¦ ¦ ^ Ring Modulation +-------- EG1 -----+ ¦ +-------- EG2 ----------------------+

2. Oscillator Section (VCO 1 & VCO 2)

Naive digital sawtooth and pulse waveforms suffer from severe Nyquist aliasing. To preserve authentic analog brightness without aliasing artifacts, implement PolyBLEP (Polynomial Band-Limited Step) residual correction around phase discontinuities.

PolyBLEP Residual Function: For phase accumulator φ ∈ [0, 1) and phase increment Δt = f₀ / fₛ: BLEP(t, Δt) = (t / Δt) - (t² / (2 · Δt²)) - 0.5 for 0 ≤ t < Δt BLEP(t, Δt) = (t / Δt) + (t² / (2 · Δt²)) + 0.5 for -Δt < t ≤ 0 BLEP(t, Δt) = 0 otherwise

VCO Waveforms & Ring Modulation

VCO 1: Triangle (integrated square or PolyBLAMP), Sawtooth (y_naive(φ) = 2φ - 1 minus BLEP(φ, Δt)), Variable Pulse/Square (PWM via dual BLEP subtraction), and White/Pink Noise.

VCO 2: Sawtooth, Square, and Narrow Pulse with independent pitch offset and master pitch tune.

Ring Modulation: In the MS-20, Ring Mod is an XOR-based digital logic network (TC4011 CMOS), NOT analog 4-quadrant multiplication. It calculates the sign product: RingOut = sgn(VCO1_Pulse) × sgn(VCO2_Pulse).

3. The Dual KORG35 Filter (VCF HPF & LPF)

The defining sonic characteristic of the MS-20 (Rev. 1) is the paired Sallen-Key KORG35 topology: an active 2nd-order High-Pass Filter followed directly in series by an active 2nd-order Low-Pass Filter. Each filter provides 12 dB/octave attenuation. Conventional unit-delay feedback (z⁻¹) breaks down and destabilizes near self-oscillation. The filter must be implemented using Topology-Preserving Transform (TPT) with trapezoidal bilinear integration.

TPT Sallen-Key Equations (Zero-Delay Feedback): Pre-warped Cutoff: ωₐ = 2π · f_c, g = tan(ωₐ / (2 · fₛ)), G = g / (1 + g) Loop Resolution: u[n] = x[n] - k · f_sat(y₂[n]) where f_sat(v) = tanh(v) State Updates: v₁ = G · (u[n] - s₁), y₁ = v₁ + s₁, s₁ ← y₁ + v₁ v₂ = G · (y₁ - s₂), y₂ = v₂ + s₂, s₂ ← y₂ + v₂ HPF Output Node: y_HP = u[n] - 2 · y₁ + y₂

4. Envelope Generators (EG1 & EG2)

EG1 (DAR - Delay, Attack, Release): Triggered by key press. Delay phase counter, followed by exponential Attack, and exponential Release upon note-off.

EG2 (HADSR - Hold, Attack, Decay, Sustain, Release): Main envelope generator featuring a Hold phase holding base level before attack, exponential attack toward target, decay toward sustain level, and exponential release.

Exponential Asymptotic Update: y[n] = y[n-1] + α · (V_target - y[n-1]), where α = 1 - exp(-1 / (τ · fₛ)).

5. Component-Level IC & Circuit Breakdown

5.1 KORG35 Hybrid Module (KLM-128 Board)

Custom 8-pin SIP thick-film hybrid. Operates as an unbuffered variable-gain transconductance cell based on a matched NPN differential pair (equivalent to 2SC945 / 2N3904) with a common current-sink transistor, driven by a control current I_bias (0 to 500 μA) at Pin 6. The output is a single-ended PNP emitter follower (2SA733 / 2N3906). The Peak feedback loop incorporates diode/transistor junctions causing asymmetric soft-clipping and harmonic enrichment without losing low-frequency gain.

5.2 Logic & Operational Amplifier ICs

TC4011BP (Quad 2-Input CMOS NAND): Implements the XOR logic network for VCO Ring Modulation and ESP pulse shaping.

TC4001BP (Quad 2-Input CMOS NOR): Manages S-Trig (active-low switch trigger) inversion and envelope gating pulses.

LM13600 / NJM13600: Dual OTA integrated circuit used in MS-20 Rev. 2 (KLM-307 board) featuring linearizing diodes and Darlington buffers.

LM324 & RC4558: General-purpose quad and dual op-amps configured for envelope RC timing stages and ESP frequency tracking.

6. Rust Multi-Target Architecture (Native TUI & Web/WASM)

To support both a Native TUI (via Ratatui and CPAL) and a Web target (via AudioWorklet and WebAssembly), the codebase must separate the core DSP engine into a strict pure-compute crate with zero I/O and zero dynamic memory allocations in the render path.

ms20-workspace/ ├── ms20-core/ # Pure Rust audio engine (no I/O, no UI) │ ├── Cargo.toml │ └── src/ │ ├── dsp/ │ │ ├── oscillator.rs # PolyBLEP VCOs + Ring Mod │ │ ├── korg35.rs # TPT Sallen-Key HPF & LPF │ │ ├── envelope.rs # DAR & HADSR state machines │ │ └── lfo.rs # Modulation generator │ ├── voice.rs # Voice allocation & signal flow │ ├── patchbay.rs # Modulation matrix / virtual cables │ └── engine.rs # Master synthesizer state & render buffer │ ├── ms20-tui/ # Native Terminal Application (ratatui + cpal) │ ├── Cargo.toml │ └── src/ │ ├── audio_host.rs # cpal stream callback invoking ms20-core │ └── ui/ # Knobs, patchbay view, visual scopes │ └── ms20-wasm/ # WebAssembly Target (wasm-bindgen) ├── Cargo.toml └── src/ └── lib.rs # WebAudio AudioWorklet processor wrapper

Audio Processor Trait & Lock-Free Thread Contract

pub trait AudioProcessor { fn process(&mut self, output_left: &mut [f32], output_right: &mut [f32]); fn handle_midi_event(&mut self, message: [u8; 3]); fn set_parameter(&mut self, param_id: u32, value: f32); }

Thread Boundary: In both cpal and AudioWorkletProcessor, the audio rendering callback must remain strictly lock-free. Use atomic primitives or ring buffers (e.g. crossbeam-channel or rtrb) to pass MIDI events and parameter knob changes from the TUI/Web UI thread to the audio thread.